# Bantay Drive — Backend Pseudo Code

Driver Monitoring System — Core Algorithm Documentation

# MODULE 1 — Face Detection & Feature Extraction

Source file: head_pose_service.dart

FUNCTION detectFaceFeatures(cameraFrame):

IF detector is busy: RETURN null

// Convert camera frame from YUV to NV21 format (ML Kit input)

nv21Frame <- convertYUV_to_NV21(cameraFrame)

faces <- ML_Kit_FaceDetector.process(nv21Frame)

IF no faces found: RETURN null

face <- faces[0]  // use the closest/first detected face

// -- Head Position --

faceCenter  <- face.boundingBox.center

normalizedX <- (faceCenter.x / frameWidth)  * 2 - 1   // range -1 to 1

normalizedY <- (faceCenter.y / frameHeight) * 2 - 1

// -- Head Angles --

pitch <- face.headEulerAngleX   // up/down tilt

yaw   <- face.headEulerAngleY   // left/right turn

roll  <- computeRollFromEyeLandmarks(face.leftEye, face.rightEye)

// -- Eye Openness (EAR) --

// ML Kit gives 0=closed, 1=open -> map to EAR scale (0.05 to 0.40)

earL <- face.leftEyeOpenProbability  x 0.35 + 0.05

earR <- face.rightEyeOpenProbability x 0.35 + 0.05

// -- Mouth Opening (MAR) --

mouthWidth    <- distance(mouthLeft, mouthRight)

mouthVertical <- distance(mouthBottom, upperLip)

mar <- mouthVertical / mouthWidth   // 0=closed, >0.5=yawning

// -- Exponential Smoothing (reduces noise between frames) --

smoothedEarL <- 0.4 x earL + 0.6 x prevEarL

smoothedEarR <- 0.4 x earR + 0.6 x prevEarR

smoothedMar  <- 0.5 x mar  + 0.5 x prevMar

smoothedRoll <- 0.5 x roll + 0.5 x prevRoll

RETURN (normalizedX, normalizedY, pitch, yaw, smoothedRoll,

smoothedEarL, smoothedEarR, smoothedMar)

# MODULE 2 — Frame Preprocessing

Source file: tflite_service.dart

FUNCTION preprocessFrame(cameraFrame, faceFeatures):

// -- Spatial Input: convert YUV -> RGB -> resize to 224x224 --

rgbImage   <- convertYUV_to_RGB(cameraFrame)

resized    <- bilinearInterpolation(rgbImage, targetSize=224x224)

normalized <- (resized / 127.5) - 1.0   // pixel range [-1.0, 1.0]

// -- Temporal Features (20 values per frame) --

features[0] = earL

features[1] = earR

features[2] = (earL + earR) / 2     // average EAR

features[3] = min(earL, earR)       // worst eye

features[4] = mar

features[5] = pitch

features[6] = yaw - 35 degrees      // compensate for 35 degree side-mount offset

features[7] = roll

// features[8..19] = sentinel (unused slots)

RETURN (normalized, features)

# MODULE 3 — AI Model Inference

Source file: tflite_service.dart

FUNCTION runInference(cameraFrame):

IF model not loaded OR already running: RETURN null

IF time since last inference < 200ms:   RETURN null   // max 5 FPS

faceFeatures             <- getCurrentFaceData()  // from HeadPoseService

(spatialInput, features) <- preprocessFrame(cameraFrame, faceFeatures)

// -- Temporal Buffer (sliding window of last 30 frames) --

shiftLeft(temporalBuffer)              // drop oldest frame

temporalBuffer[last] <- features       // add current frame

// -- Compute Window Statistics (over 30-frame window) --

earMean  <- average of valid EAR values in buffer

earMin   <- minimum EAR in buffer

marMax   <- maximum MAR in buffer

marAbove <- fraction of frames where MAR > 0.5 (yawning threshold)

earTrend <- linear regression slope of EAR (is eye closing over time?)

// -- Normalize Temporal Input --

FOR each frame in temporalBuffer:

normalizedFeatures <- (rawValue - mean) / scale

// -- Run HybridNet V3 Model --

outputs <- DMS_HybridNet_V3(

temporalInput = normalizedTemporalBuffer,   // 30 frames x 25 features

spatialInput  = spatialInput                // 224x224x3 image

)

// -- Parse 3 Outputs --

behaviorProbs <- softmax(outputs.behavior)   // 13 behavior classes

parentProbs   <- outputs.parent              // 3 parent states

gazeProbs     <- outputs.gaze                // 8 gaze zones

parentClass <- argmax(parentProbs)   // 0=NATURAL, 1=DISTRACTED, 2=DROWSY

gazeZone    <- argmax(gazeProbs)     // e.g., ROAD, LAP, LEFT, RIGHT...

RETURN classifyState(behaviorProbs, parentClass, gazeZone)

# MODULE 4 — State Classification

Source file: tflite_service.dart -> _buildResult()

FUNCTION classifyState(behaviorProbs, parentClass, gazeZone):

// -- Group 13 classes into 3 categories --

neutralPct    <- sum of probs for class 0 (safe), class 1 (talking)

drowsyPct     <- sum of probs for classes 9-12 (yawning, fatigue, microsleep)

distractedPct <- sum of probs for classes 2-8 (phone, texting, radio, etc.)

bestDrowsyClass     <- class with highest score among drowsy group

bestDistractedClass <- class with highest score among distracted group

// -- False Positive Suppression --

IF bestDistractedClass == "grooming" AND score < 55%:

IF 2nd-best distracted score is within 20% of grooming:

suppress grooming -> use 2nd-best instead  // side-mount artifact

IF bestDistractedClass == "distracted_body" AND score < 72%:

IF a more specific distracted class is close behind:

use more specific class instead

// -- Gaze Compensation for 35 degree Side Mount --

compensatedYaw   <- faceYaw - 35 degrees

driverOnRoad     <- |compensatedYaw| <= 25 degrees

parentDistracted <- (parentClass == DISTRACTED)

// -- Raw State Decision (3-stage distraction gate) --

IF drowsyPct >= 15% AND bestDrowsyScore meets minimum:

rawState <- "drowsy"

ELSE IF distractedPct >= 40% AND bestDistScore >= 25%:

rawState <- "distracted"  // Stage 1: strong signal, no extra gate needed

ELSE IF distractedPct >= 22% AND bestDistScore >= 12%

AND parentDistracted AND class != body:

rawState <- "distracted"  // Stage 2: moderate, requires parent model

ELSE IF distractedPct >= 15% AND bestDistScore >= 8%

AND parentDistracted AND class != body AND NOT driverOnRoad:

rawState <- "distracted"  // Stage 3: weak signal, also off-road required

ELSE:

rawState <- "neutral"

// -- Consecutive-Frame Debounce --

IF rawState == "drowsy":     consecutiveDrowsy++

IF rawState == "distracted": consecutiveDistracted++

IF rawState == "neutral":    decrement both counters (floor 0)

// -- Final Confirmed State --

IF consecutiveDrowsy >= 3:

RETURN state="drowsy"

ELSE IF consecutiveDistracted >= threshold (6 / 12 / 22 based on stage):

RETURN state="distracted"

ELSE:

RETURN state="neutral"

# MODULE 5 — Alert System

Source file: monitor_screen.dart

FUNCTION onModelOutput(inferenceResult):

UPDATE UI: alertness%, drowsiness%, distraction% displays

SAVE frame state to database (per session)

MATCH inferenceResult.state:

CASE "drowsy":

consecutiveDrowsy++

consecutiveDistracted <- max(0, consecutiveDistracted - 1)

CALL checkAndTriggerAlert("DROWSY", consecutiveDrowsy)

CASE "distracted":

consecutiveDistracted++

consecutiveDrowsy <- max(0, consecutiveDrowsy - 1)

CALL checkAndTriggerAlert("DISTRACTED", consecutiveDistracted)

CASE "neutral":

decrement both counters (floor 0)

IF alertLevel is 1 or 2 AND both counters below L1 threshold:

CLEAR alert, stop sound

FUNCTION checkAndTriggerAlert(type, consecutiveCount):

// Sensitivity settings (user configurable):

//   Low=0:    thresholds [5, 10, 15]

//   Medium=1: thresholds [3,  6,  9]  <- default

//   High=2:   thresholds [2,  4,  6]

thresholds <- getSensitivityThresholds(userSetting)

IF consecutiveCount < thresholds[0]: RETURN  // not enough frames yet

newLevel <- 1

IF consecutiveCount >= thresholds[2]: newLevel <- 3

IF consecutiveCount >= thresholds[1]: newLevel <- 2

IF newLevel <= currentAlertLevel: RETURN  // do not downgrade

currentAlertLevel <- newLevel

SHOW alert banner on screen

SAVE alert event to database (type, level, timestamp)

IF newLevel == 1:

PLAY short warning sound (once)

IF newLevel == 2:

PLAY warning sound (repeat up to 3 times)

SAVE video clip of the incident

IF newLevel == 3:

PLAY critical alarm (looping, until dismissed)

SAVE video clip of the incident

# Overall System Flow

LOOP every 200ms (while monitoring is active):

cameraFrame <- getCameraFrame()

faceData    <- detectFaceFeatures(cameraFrame)        // Module 1

features    <- preprocessFrame(cameraFrame, faceData) // Module 2

result      <- runInference(features)                 // Module 3

state       <- classifyState(result)                  // Module 4

onModelOutput(state)                                  // Module 5